REPORT ON PROJECT PHASE II

“Comparative study for pattern analysis and

Recognition of musical segments”

SUBMITTED to SAVITRIBAI PHULE PUNE UNIVERSITY

FOR PARTIAL FULFILLMENT

OF THE REQUIREMENTS for the DEGREE OF

BACHELOR OF ENGINEERING

in

Electronics and Telecommunication

By

PANKAJ SINGH (B120053156)

MANALI MOHITE (B120053138)

JATIN HEDAOO (B120053080)

Department Of Electronics and TELECommunication

PUNE INSTITUTE OF COMPUTER TECHNOLOGY

PUNE – 43

JUNE 2017

Department of Electronics and Telecommunication Engineering

Pune Institute of Computer Technology, Pune – 43

CERTIFICATE

This is to certify that the Project Phase II Report entitled “Comparative study for pattern analysis and  Recognition of musical segments” has been successfully completed by

Pankaj Singh

Manali Mohite

Jatin Hedaoo

towards the partial fulfillment of the degree of Bachelor of Engineering in Electronics and Telecommunication as awarded by the Savitribai Phule Pune University, at Pune Institute of Computer Technology during the academic year 2016-17.

Ms. A.M. Kulkarni					                                      Prof. Dr. Y. Ravinder

Project Guide								   HOD, E&TC Dept.

ACKNOWLEDGEMENT

The completion of this project and report could not have been possible without the participation and assistance of so many people whose names may not all be enumerated. Their contributions are sincerely appreciated and gratefully acknowledged.

I express my deep sense of gratitude to Ms. A M Kulkarni (Project Guide) for her valuable guidance, keen interest and encouragement at various stages in completion of this project.

I would like to express my sincere gratitude to the Management of this institute and to our Principal, Dr. P T Kulkarni, project coordinator Dr.R.C. Jaiswal and HOD, E&TC Dept. Dr.Y.Ravinder, Pune Institute of Computer Technology, Pune, for their constant support and encouragement in the preparation of this report and for making available library and laboratory facilities needed to prepare this report.

Last but not least, we would like to express our gratitude to our friends and respondents for support and willingness to be there with us, to share their support, either morally, financially and physically, thank you.

PANKAJ SINGH

MANALI MOHITE

JATIN HEDAOO

CONTENTS

|  | Abstract |  | i |
| --- | --- | --- | --- |
|  | List of Symbols |  | ii |
|  | List of Acronyms |  | iii |
|  | List of Figures |  | iv |
|  | List of Tables |  | v |
| 1 | Introduction |  | 1-6 |
|  | 1.1 | Background | 1 |
|  | 1.2 | Relevance | 1 |
|  | 1.3 | Literature Survey | 2 |
|  | 1.4 | Comparison of audio signals | 5 |
|  | 1.5 | Motivation |  |
| 2 | System design and requirements |  | 7-10 |
|  | 2.1 | Hardware and Software Requirements | 7 |
|  | 2.2 | Functional Requirements | 7 |
|  | 2.3 | Non-Functional Requirements | 8 |
|  | 2.4 | Block Diagram Details | 8 |
| 3 | Algorithm and implementation |  | 11-16 |
|  | 3.1 | Note Detection Algorithm | 11 |
|  | 3.2 | Comparison Algorithm using dynamic programming | 15 |
| 4 | Screenshots of the project |  | 17-19 |
| 5 | Progress of project and discussion |  | 20-23 |
| 6 | Conclusion |  | 24 |
| 7 | Future plan |  | 25 |
|  | References |  | 26 |

ABSTRACT

In this project, we are dealing with the analysis and recognition of musical segments inside any musical audio file given to the software.

We would have a basic database of all the segments or patterns that are generally observed. We would then give in any audio input signal to the software, now the system will detect the musical notes present in the input signal. Then we apply the fast fourier transform algorithm to each detected segment or note in order to find the fundamental frequency. Now once we get the fundamental frequency then it used to convert to particular note name corresponding to that frequency. This will result in getting the order in which the notes are present in the audio file.

The other approach that we considered is using chromagram. We plotted the chromagram of the input signal. Using this we can clearly analyze what musical notes are present at a particular time interval.

| List of Symbols |  |
| --- | --- |
| f | Frequency |
| n | number of semitones above C4 |

II

| List of Acronyms |  |
| --- | --- |
| .wav | Audio files |
| RIFF | Resource Interchange File Format |
| LPCM | linear pulse-code modulation |
| FFT | Fast fourier transform |

III

| List of Figures |  |  |
| --- | --- | --- |
| Fig 1 | a) Musical score of a C-major scale. (b) Chromagram obtained from the score. (c) Audio recording of the C-major scale played on a piano. (d) Chromagram obtained from the audio recording. | 3 |
| Fig 2 | Generalized suffix tree for the strings "ABAB", "BABA" and "ABBA", numbered 0, 1 and 2. | 5 |
| Fig 3 | Comparative study for pattern analysis and recognition of musical segments (Block diagram) | 7 |
| Fig 4 | Sample audio input | 9 |
| Fig 5 | Power spectrum | 10 |
| Fig 6 | Audio input | 11 |
| Fig 7 | Audio processing | 12 |
| Fig 8 | Note names from frequency | 13 |
| Fig 9 | m*n matrix for storing the solutions of substrings | 15 |
| Fig 10 | FFT of signal 1 | 21 |
| Fig 11 | Chromagram | 22 |
| Fig 12 | Short term FT | 23 |

IV

| List of Tables |  |  |
| --- | --- | --- |
| Table 1 | Note frequency chart | 14 |

V

CHAPTER 1

Introduction

  - Background

In this project, we would be dealing with the analysis and recognition of musical segments inside any given sample of input. We would have a basic database of all the segments or patterns that are generally observed regularly or most often. We then would give in any sound input to the system. Now the system will compare the input signal with the already present database. This will result in matching of the pattern segments between the input and the already existing database. Hence we would have had compared and analyzed the input and the input can be modified in accordance with the already existing database.

  - Relevance

Musical analysis is used effectively for understanding of how tonal (and other forms of music) are constructed.

Understanding these underlying structures can help performers in several ways, for instance, with the phrasing of music. By understanding how the underlying phrase structures work, how complete and incomplete structures can be joined, it is possible to understand how feelings of expectation and realization, tension and resolution can be created in music.

Understanding how this process works can help with the phrasing and expression of the music. Understanding the structure of music from different periods is an important skill for composers. Whilst the theoretical and analytical ideas in this book are not intended to be a compositional method, composers do need to understand how music is structured in order to create music with continuity, movement and shape. By understanding concepts such as: root progression patterns, surface voice leading patterns, static and dynamic harmony and how they relate to each other and vary, style by style, the composer can use these ideas and adapt them to any style he or she wishes.

Possibly the most important reason is curiosity. Music has a language that people all over the world can understand at a subconscious level. Large numbers of people round the world listen to music that is tonal or utilizes tonal harmony and tonal structures. Tonal harmony has spread and influenced music all around the world. So it is worthwhile trying to understand how this widespread language works and how it varies. It is worth trying to understand the syntax of tonal music and the structure of individual pieces of music simply because listening to it is important to so much of humanity. It is one of those big human questions.

  - Literature Survey

Pattern Discovery Techniques [3] for Music Audio Human listeners are able to recognize structure in music through the perception of repetition and other relationships within a piece of music. This work aims to automate the task of music analysis. Music is “explained” in terms of embedded relationships, especially repetition of segments or phrases. The step in this process are the transcription of audio into a representation with a similarity or distance metric, the search for similar segments, forming clusters of similar segments, and explaining music in terms of these clusters. Several transcription methods are considered: monophonic pitch estimation, chroma (spectral) representation, and polyphonic transcription followed by harmonic analysis. Also, several algorithms that search for similar segments are described. These techniques can be used to perform an analysis of musical structure, as illustrated by examples. Given the current state of the art,virtually any technique that reveals structure in an audio recording is interesting. Techniques such as beat detection, key detection, chord identification, monophonic and polyphonic transcription,melody and bass line detection, source separation, speech recognition, and instrument identification all derive some higher level information from music audio. There is some hope that by continuing to develop these techniques and combine them, we will be better able to reason about, search, and classify music, starting from an audio representation.

In this work, we examine ways to discover patterns in music audio and to translate this into a structural analysis. The idea is quite simple: musical structure is signaled by repetition. Of course, “repetition” means similarity at some level of abstraction above that of audio samples. We must process sound to obtain a higher-level representation before comparisons are made, and must allow approximate matching to allow for variations in performance, orchestration, lyrics, etc. In a number of cases, our techniques have been able to describe the main structure of music compositions.

Chroma Feature:

Chroma features are an interesting and powerful representation for music audio in which the entire spectrum is projected onto 12 bins representing the 12 distinct semitones (or chroma) of the musical octave. Since, in music, notes exactly one octave apart are perceived as particularly similar, knowing the distribution of chroma even without the absolute frequency (i.e. the original octave) can give useful musical information about the audio -- and may even reveal perceived musical similarity that is not apparent in the original spectra.

One main property of chroma features is that they capture harmonic and melodic characteristics of music, while being robust to changes in timbre and instrumentation.

Figure 1 : a) Musical score of a C-major scale. (b) Chromagram obtained from the score. (c) Audio recording of the C-major scale played on a piano. (d) Chromagram obtained from the audio recording.

The underyling observation is that humans perceive two musical pitches as similar in color if they differ by an octave. Based on this observation, a pitch can be separated into two components, which are referred to as tone height and chroma. Assuming the equal-tempered scale, one considers twelve chroma values represented by the set

{C, C♯, D, D♯, E ,F, F♯, G, G♯, A, A♯, B}

that consists of the twelve pitch spelling attributes as used in Western music notation. Note that in the equal-tempered scale different pitch spellings such C♯ and D♭ refer to the same chroma. Enumerating the chroma values, one can identify the set of chroma values with the set of integers {1,2,...,12}, where 1 refers to chroma C, 2 to C♯, and so on. A pitch class is defined as the set of all pitches that share the same chroma. For example, using the scientific pitch notation, the pitch class corresponding to the chroma C is the set

{..., C−2, C−1, C0, C1, C2, C3 ...}

consisting of all pitches separated by an integer number of octaves. Given a music representation (e.g. a musical score or an audio recording), the main idea of chroma features is to aggregate for a given local time window (e.g. specified in beats or in seconds) all information that relates to a given chroma into a single coefficient. Shifting the time window across the music representation results in a sequence of chroma features each expressing how the representation's pitch content within the time window is spread over the twelve chroma bands. The resulting time-chroma representation is also referred to as chromagram. The figure above shows chormagrams for a C-major scale, once obtained from a musical score and once from an audio recording. Because of the close relation between the terms chroma and pitch class, chorma features are also referred to as pitch class profiles.

The basic formula for the frequencies of the notes of the equal tempered scale is given by
fn = f0 * (a)n , where f0 = the frequency of one fixed note which must be defined. A common choice is setting the A above middle C (A4) at f0 = 440 Hz.
n = the number of half steps away from the fixed note you are. If you are at a higher note, n is positive. If you are on a lower note, n is negative.
fn = the frequency of the note n half steps away.
a = (2)1/12 = the twelth root of 2 = the number which when multiplied by itself 12 times equals 2 = 1.059463094359...

  - Comparison of audio signals:

In computer science, the longest common substring problem is to find the longest string (or strings) that is a substring (or are substrings) of two or more strings.

In our project, we used this to compare the notes of different audio signals.

Algorithms:

Given two strings, {\displaystyle S} S of length m{\displaystyle m} and {\displaystyle T} T of length n {\displaystyle n}, find the longest strings which are substrings of both {\displaystyle S} S and {\displaystyle T}T.

One can find the lengths and starting positions of the longest common substrings of S {\displaystyle S} and T {\displaystyle T} in {\displaystyle \Theta (n+m)} time O(m+n) with the help of a generalized suffix tree. Finding them by dynamic programming costs O(nm) {\displaystyle \Theta (nm)}.

Suffix Tree:

Fig 2. Generalized suffix tree for the strings "ABAB", "BABA" and "ABBA", numbered 0, 1 and 2.

The longest common substrings of a set of strings can be found by building a generalized suffix tree for the strings, and then finding the deepest internal nodes which have leaf nodes from all the strings in the subtree below it. The figure on the right is the suffix tree for the strings "ABAB", "BABA" and "ABBA", padded with unique string terminators, to become "ABAB$0", "BABA$1" and "ABBA$2". The nodes representing "A", "B", "AB" and "BA" all have descendant leaves from all of the strings, numbered 0, 1 and 2.

Building the suffix tree takes {\displaystyle \Theta (N)}O(N) time (if the size of the alphabet is constant). If the tree is traversed from the bottom up with a bit vector telling which strings are seen below each node, the k-common substring problem can be solved in {\displaystyle \Theta (NK)}O(NK) time.

### Dynamic programming:

Dynamic programming (also known as dynamic optimization) is a method for solving a complex problem by breaking it down into a collection of simpler subproblems, solving each of those subproblems just once, and storing their solutions. The next time the same subproblem occurs, instead of recomputing its solution, one simply looks up the previously computed solution, thereby saving computation time at the expense of a (hopefully) modest expenditure in storage space. (Each of the subproblem solutions is indexed in some way, typically based on the values of its input parameters, so as to facilitate its lookup.) The technique of storing solutions to subproblems instead of recomputing them is called "memoization"

  - Motivation

Musical genres are categorical labels created by humans to characterize pieces of music. A musical genre is characterized by the common characteristics shared by its members. These characteristics typically are related to the instrumentation, rhythmic structure, and harmonic content of the music. Genre hierarchies are commonly used to structure the large collections of music available on the Web. Currently musical genre annotation is performed manually. Automatic musical genre classification can assist or replace the human user in this process and would be a valuable addition to music information retrieval systems. In addition, automatic musical genre classification provides a framework for developing and evaluating features for any type of content-based analysis of musical signals.The automatic classification of audio signals into an hierarchy of musical genres is explored. More specifically, three feature sets for representing timbral texture, rhythmic content and pitch content are proposed. The performance and relative importance of the proposed features is investigated by training statistical pattern recognition classifiers using real-world audio collections. Both whole file and real-time frame-based classification schemes are described. Using the proposed feature sets, classification of 61% for ten musical genres is achieved. This result is comparable to results reported for human musical genre classification.

CHAPTER 2

SYSTEM DESIGN AND REQUIREMENTS

2.1 Hardware and Software Requirements

2.1.1 Overall Description

The project requires you to use MATLAB Software as the operating environment. The User Interface is easy to handle. User needs to enter the audio file name in the input section. This section reads the audio file with the given name in the MATLAB project directory using ’audioread’ function which is only available in the latest releases(after 2011).So the user should make sure that the MATLAB software he/she is running must be up to date. The project is built in MATLAB R2014a Windows version. It portrays important graphs for advanced anlaysis if required by the user so it is important that the version also contains the basic libraries which are provided by default by the version on which the project is built.

2.1.2 External Interface Requirement

The UI is developed using MATLAB Graphical User Interface (GUI).It takes the input audio

file name and to locate and store the file for processing. The AUTO mode uses intelligent note

detection system to find the points where there is a note change. The MANUAL mode allows the user to select the threshold of intensity above which the notes will be detected. The run-time for the latter is more compared to the former.After the note variations are compiled together the,

PLAY NOTES button creates a new audio file and writes the generated audio file to it. The UI

also provides advanced features like plots of power spectrum of each note, note name according to standard sheet music notation,etc.

2.2  Functional Requirements

- The system should process the input given by the user only if it is an audio file (.wav file etc.)

- System shall compare the given input audio files when the input given is in the required format.

- System should detect same pattern present in the audio files.

2.3  Non-Functional Requirements

- The analyzing method  is very easy to implement and compare two audio files.

2.4  Block Diagram Details

The basic flow diagram of the project is given below and each and every block is explained in details later. Audio files are taken as input for analysis and recognition of different patterns. This will give a same or repeated pattern in given audio input files.

Figure 3: Comparative study for pattern analysis and recognition of musical segments

Each block is explained in detail below:

2.4.1: Audio input files

An audio signal is a representation of sound, typically as an electrical voltage. Audio signals have frequencies in the audio frequency range of roughly 20 to 20,000 Hz (the limits for human hearing). Audio signals may be synthesized directly, or may originate at a transducer such as a microphone, microphone instrument, phonogram cartridge, or tape. Loudspeakers or headphones  convert an electrical audio signal into sound. Digital representations of audio signals exist in a variety of formats. In our project we are using audio files in .wav format for analysis.

The WAV acronym stands for Waveform Audio Format which is a high-quality _le format used for storing an audio bit stream. It was introduced in 1991 by Microsoft and IBM for use on the Windows 3.1 operating system and is used for applications such as CDs. The format

is based on the Resource Interchange File Format (RIFF), this stores chunks and sub-chunks

which are each indexed. The usual bit stream encoding is the linear pulse-code modulation (LPCM) format.

2.4.2 Pre-Processing:

After acquiring of the audio signal, it will be processed through sequence of preprocessing steps to be ready for the next step. These steps include noise removal, FFT, breaking of audio files into segment.

Noise removal involves reducing noise in an audio signal using filter in Matlab. Audio files are prone to a variety of types of noise. Noise is the result of errors in the audio file analysis process, which will help in recognition of the pattern.

Sampling Theorem:

In order to perform any form of processing by digital computers, the signals must

be reduced to discrete samples of a discrete-time domain. The operation that transforms

a signal from the continuous time to the discrete time is called sampling, and it

is performed by picking up the values of the continuous-time signal at time instants

that are multiple of a quantity T, called the sampling interval. The quantity Fs = 1/T

is called the sampling rate.

Discrete Fourier transform:

In mathematics, the discrete Fourier transform (DFT) converts a finite list of

equally spaced samples of a function into the list of coefficients of a finite combination

of complex sinusoids, ordered by their frequencies, that has those same

sample values. It can be said to convert the sampled function from its original domain

(often time or position along a line) to the frequency domain. The input samples

are complex numbers (in practice, usually real numbers), and the output coefficients

are complex as well. The frequencies of the output sinusoids are integer multiples

of a fundamental frequency, whose corresponding period is the length of the sampling

interval. The combination of sinusoids obtained through the DFT is therefore periodic with that same period.

fig 4:Sample Audio signal

Fig 5:Power Spectrum

2.4.3. Analyzing Process:

Step by step execution of an audio signal, used to convey the inputs, outputs, and operations that take place during each phase. A process analysis can be used to improve understanding of how the process operates, and to determine potential targets for process improvement through removing waste and increasing efficiency.

2.4.4 Comparison:

In order to get similarity between any two given audio input files, we used dynamic programming to compare the detected notes from the audio file.

The comparison result shows the longest similar segment in the given audio files.

2.4.6 Software (MATLAB):

Matlab (matrix laboratory) is a multipurpose environment and a proprietary programming language developed by MathWorks, MATLAB allows matrix manipulations, plotting of functions and data, implementation of algorithms, creation of user interfaces, and interfacing with programs written in other languages, including C, C++, C#, Java.

CHAPTER 3

ALGORITHM AND IMPLEMENTATION

3.1  Note Detection Algorithm:

The project follows the following implementation. Input audio file which is in the project directory is taken as read and converted into an array with one or two streams. If it contains two streams then they are merged into one by taking average of the two using the following formula.

y=(y(:,1)+y(:,2))/2;

The array stores the audio signal with sampling frequency 44100 samples per second.

Fig 6:   Input

Audio processing involves two types of note detection. Only the regions where a note is present is taken for processing. Once a note region is isolated we apply Fast Fourier Transform to the region and analyse the data by plotting the power spectrum of the signal.

Then the frequency is estimated using two different fast algorithms and the appropriate frequency is taken.

Figure 7:   Audio Processing

h = waitbar(0,'Please wait...');

points = timeout(y,fs);

note=2;factor=1;

zz=y*0;

str_prev='';

final=' ';

while(note<length(points))

x=y(points(note):points(note+1));

[freq,amp]=main_func(x);

s=freqtonote(freq);

if(note~=2)

str=strcat(',',s);

else

str=s;

end

if(~(strcmp(str_prev,s)))

final=strcat(final,str);

end

str_prev=s;

note=note+1;

waitbar(note/length(points))

end

if(strcmp(final,' '))

final='null';

end

output1='C:\Users\Pankaj Singh\Documents\MATLAB\project_new\output_notes\';

output1=strcat(output1,strcat(filename,'.txt'));

setappdata(0,'output1',output1);

fileID=fopen(output1,'w');

fprintf(fileID,'%s',final);

fclose(fileID);

close(h)

The fundamental frequency is calculated directly by using maximum power point in the power spectrum and by taking the difference between consecutive harmonics. The frequency thus obtained is used to calculate the note name of according to the standard sheet music notation.

Figure 8:   Note name from Frequency

Table 1: Note frequency chart

Formula to convert frequency to note:

n = round (12*log(f/260)/log(2))

Where n is number of semitones above C4,

And f is frequency to be converted to respective note.

3.2   Comparison Algorithm using dynamic programming:

Naive Approach:

Check all the substrings from first string with second string anxd keep track of the maximum.

Time Complexity: O(n2*m), O(n2) for the substring and O(m) for check all the substrings with second string.

Dynamic Programming:

We used dynamic programming to solve this problem in bottom-up manner. We Created a matrix of size of m*n and stored the solutions of substrings to use them later.

Figure 9 : m*n matrix for storing the solutions of substrings

Base Cases: If any of the string is null then LCS will be 0.

Check if ith character in one string A is equal to jth character in string B

Case 1: both characters are same

LCS[i][j] = 1 + LCS[i-1][j-1] (add 1 to the result and remove the last character from both the strings and check the result for the smaller string.)

Case 2: both characters are not same.

LCS[i][j] = 0

At the end, traverse the matrix and find the maximum element in it, this will the length of Longest Common Substring.

m = length(str1);

n = length(str2);

dp=zeros(m,n);

maxLen = -999999;

suffixEnd = -1;

for  i=2:m+1

for j=2:n+1

if str1(i-1) == str2(j-1)

dp(i,j) = dp((i-1),(j-1)) + 1;

if dp(i,j) > maxLen

maxLen = dp(i,j);

suffixEnd = i;

end

else

dp(i,j) = 0;

end

end

end

s=str1(suffixEnd-maxLen:suffixEnd-1);

CHAPTER 4

SCREEENSHOTS OF THE PROJECT

4.1  Audio input:

4.2  Tempo and Audio control menu:

4.3  Output interface for the detected notes:

4.4   Output interface for audio comparison:

4.5   Complete project user interface:

CHAPTER 5

PROGRESS OF PROJECT AND DISCUSSION

5.1  Frequency estimation of a periodic wave:

We started by calculating the frequency estimation of a periodic wave. The technique

did not require us to use Fourier transform so it made things simple. Our approach

to this was the rule that frequency of a periodic signal is proportional to the

Number of maxima or minima in a fixed finite time interval.

5.2  Frequency estimation of a non-periodic wave:

After we were successful in finding the period of a periodic wave,the next challenge

was that the audio signals practically are not exactly periodic because of the

minute disturbances in the medium which cause considerable fluctuations in the audio

signal in the order of period. So we changed the approach and calculated the

period by brute-force method i.e varying period within certain limits and checking

which value of period satisfied the required conditions the most.

5.3  Frequency estimation with Fourier Analysis:

5.3.1 No Harmonics

The above method was successful but it was slow, then we finally resorted to using

the fast Fourier transform provided by MATLAB, our operating environment. But

even that was not enough because if T is the period then nT, where n=2,3,4...... can

also be the period.

5.3.2 In presence of Harmonics

Fortunately, the solution to the above problem was the problem itself. If more than

one harmonics are present then we can find the fundamental frequency by two methods

i.e by finding the least frequency in them or by calculating the difference between

two consecutive harmonics. In almost all the cases both give the same result.

But in few cases the first methods fails if the least frequency is undetected. If

something like that happens the second method ensures the correctness of the frequency.

5.4 Note Detection

Once we were able to calculate the frequency of a note the next challenge was to

isolate the note from the audio input. Since our code works only if the input contains

a single note and no other signals.

Figure 10 :    Fast fourier transform of audio input signal

Initially we limited the domain of the input and designed a note detection method which works on the fact that a note begins with fast increase in intensity and ends with decrease in intensity below a threshold. The method worked perfectly except when two notes were very close i.e when a note started before a note ended. So two notes were taken as a single note and thereby resulted in errors while detected frequency in that region. Since the note detection

was not the main agenda of our project we learned from external sources to develop

the note detection which used Gaussian filters and other advanced functions provided

by MATLAB.

Figure 11:  Chromagram of input audio signal

We also considered another approach to our problem by using chromagram.The conversion of an audio recording into a chroma representation (or chromagram) may be performed either by using short-time Fourier transforms in combination with binning strategies or by employing suitable multirate filter banks.

Figure 12:    Short time fourier transform of input audio signal

Furthermore, the properties of chroma features can be significantly changed by introducing suitable pre- and post-processing steps modifying spectral, temporal, and dynamical aspects. This leads to a large number of chroma variants, which may show a quite different behavior in the context of a specific music analysis scenario.

CHAPTER 6

CONCLUSION

We developed a system for the recognition and analysis of the input audio data files.

We concluded that this approach is efficient mainly because of its ease of implementation and the Speed with which it performs its prescribed functions. We are preprocessing the input audio file to normalize it to a prescribed level and eliminate the noise if any. When we increased the tempo of the audio signal then the accuracy of our system reduced. We also made a GUI with all the analyses that we had accounted for throughout the development of the project.This GUI is depictive of all the processing that is done on the audio files.To conclude, we did all the processing on the audio file, chose the best method i.e. the note detection method, compared the input audio file with the already present database, and got the string of notes that were repeating to get the pattern.

CHAPTER 7

FUTURE SCOPE

Following are some further improvements and limitations that can be overcome development that would enhance the system and increase its usage value:

Advanced Note Detection:

There are lot of ways we to improve and customise note detection. Most of them use variations in intensity, which is not the right way because strictly speaking a note is said to change when the frequency of the signal changes. It is not easy to keep track of change in frequency because the change is gradual and hence it is an existing challenge. Moreover the frequency estimation for calculating note detection requires note detection in a crude sense which is pavesway for development in this area.

Non Periodic Signal analysis:

The process is relatively simple if the signal were sinusoidal or periodic. But the real life musical notes or vocals are approximately periodic and the frequency itself changes with time because a sample may contain more than one note and that is how music is played. While Fourier Analysis is a nice solution to this problem it is not sufficient. Theoretically it may be sufficient but its high

level implementation is not as there is resolution and run time limit. There is scope to overcome latter by designing algorithms especially for the purpose of frequency estimation and not focusing on phase detection.

Multiple Notes at a time:

Our project assumes that only a single note is played at a time. But that is not it. We can develop it further by using Fourier Analysis again. There are existing algorithms which can isolate multiple notes. After splitting the audio sample into individual notes we can apply our own techniques to find the frequency.

REFERENCES:

Books:

[1] Robert Schalkoff, “Pattern Recognition”, Wiley

[2] C. M. Bishop, Christopher, “Pattern Recognition and Machine Learning”, Springer

[3] Shaila D. Apte, “Speech and Audio Processing”, Wiley India

Transaction/Journals:

[3] Carnegie Mellon University Research Showcase @ CMU; Computer Science Department

School of Computer Science; 10-2002 Pattern Discovery Techniques for Music Audio.

[4] P. Rao, Chapter in Speech, Audio, Image and Biomedical Signal Processing using Neural

Networks, (Eds.) Bhanu Prasad and S. R. Mahadeva Prasanna, Springer-Verlag, 2007.

[5] Mark Sandler, Finding repeating patterns in acoustic musical signals applications for audio

thumbnailing.

Websites:

[6] https://labrosa.ee.columbia.edu/matlab/chroma-ansyn/

[7] https://en.wikipedia.org/wiki/Chroma_feature

[8] https://in.mathworks.com/videos/creating-a-gui-with-guide-68979.html

[9] https://in.mathworks.com/videos/creating-a-gui-with-guide-68979.html

[10] https://en.wikipedia.org/wiki/Short-time_Fourier_transform